TECHNICAL SPECIFICATIONS

Optics

- 13-element 40:1 zoom high-resolution optic system

- Beam aperture: 1.1° to 18° and 2.2° to 45°

- Tempered anti-reflective coated lenses

- 160 mm frontal lens

Light source

- Lux at 10 m, over 1,340,000 lx (narrow angle)

- Colour temperature output: 6800 K

- Rated life (L70): up to 12,000 hours

- Speckle-free source

Movement

- Extremely accurate positioning

- Moving-head operated via either 8- or 16-bit resolution

- Pan and tilt automatic repositioning

- Moving-head range: infinite (pan & tilt)

Colours

- Sophisticated colour mix system providing CMY colour mixing

- Variable CTO colour temperature correction 3000 to 6800 K

- Fixed colour wheel with 15 complementary colours, 1 multicolour animation section and 1 gobo correction filter (1/4 CTB)

Gobos

- One indexable rotating gobo wheels with 10 Quartz glass gobos (or metal), plus open position

- Adjustable-speed rotating gobo in both directions

- Fixed multi-position metal gobo wheel with instant access to 25 metal gobos, 4 beam reducers and open position

- Gobo diameter (rotating): 16 mm

- Image diameter (rotating): 10 mm

- Gobo thickness (rotating): 1.1 mm

Frost

- Light frost (1°) to soften the edge of the beam

- Heavy frost (5°) for Wash effects

Effects

- Focusable graphic animation effect-wheel with continuous rotation in both directions (on fixed gobo wheel)

- 2 set of 3 combinable rotating and indexable prisms

Dimmer / Strobe

- Electronic dimmer, allowing perfect light adjustment from 0 to 100% without colour variation

- Strobe effect, with speed adjustment from 1 to 25 flashes per second

Software features

- Local DMX addressing of luminaire and optional parameters through its built-in LCD control panel

- Remote DMX addressing of luminaire and optional parameters through a standard RDM DMX controller

- Information menu including hour counter, temperature, software version

Hardware features

- Graphic LCD display for addressing and special functions settings, with flip function

- 5 menu buttons to set functions

- Excess temperature protection

- Integrated wireless CRMX TiMo RDM receiver from LumenRadio

- IP65 XLR 5 pin male and female connectors for DMX connection

- IP65 RJ45 IN / OUT connectors for ArtNet and sACN connection

- IP65 Power connectors male and female for power connection

Control

- DMX 512 protocol, through IP65 DMX cable or a wireless system

- DMX-RDM compatible

- Stand-alone mode and Main/Follow modes

- ArtNet & sACN protocol through Ethernet cable

- Local control panel, with IP65 LCD display

- Choice of 1 DMX mode (43 DMX channels)

Power supply

- Electronic power supply with active PFC

- 100 to 240 Volts – 50/60Hz

- Power: 660 W maximum

- Power: 550 W typical at 230 V

Cooling system

- Advanced high-performance cooling system

- IP68 Self-adjusting variable speed fans for quiet operation (Auto mode)

- Selectable ventilation user modes including Super Silent Mode

- Safety protection against excess temperature

Housing

- Moving-head skeleton made of aluminium metal plates

- Head in die-cast aluminium

- Base in die-cast aluminium

- Heatsinks in aluminium and copper

- Arms covers in self-extinguishing fire retardant ABS PC (V0 class)

- Four-side handles for transportation

- Four heavy-duty feet for better stability

- IP65 protection rating

- Exterior finish: black (Carbon)

Installation

- Fastening bracket system: two Omega ¼ turn brackets designed for use with standard clamps

- Mounting points: four ¼ turn locking fittings allowing installation of Omega brackets on luminaire

- Safety cable attachment point

Operating parameters

- Operating positions: all (device on floor or fixed to a support)

- Maximum permitted ambient temperature (Ta max): 45° C (113°F)

- Minimum permitted ambient temperature (Ta min): -20° C (-4°F)

- Minimum usage distance: 25 m (82ft)

Compliance

EU (EMC & LVD):

- EMC Directive 2014/30/EU

- LVD Directive 2014/35/EU

US (Safety & EMC)

- UL 1573

- FCC Part 15, Subpart B:2023

Substance régulations

- RoHS

- REACH

- CP65

Size

- Product: 339 x 592 x 297 mm (l x h x d)

- Flight-case: 395 x 630 x 335 mm (l x h x d)

Weight

- Product: 25.4 kg

Product codes

- 014350: RAWBEAM 350 - LASER SOURCE IP65 3 SERIES

Copyright © 2001-2026 Ayrton SAS All rights reserved

Specifications subject to change without notice.